| ONE | PIPER ARROW IV EMERGENCY PROCEDURES | ONE |
| --- | --- | --- |

| ENGINE FIRE DURING START |  |
| --- | --- |
| Starter | CONTINUE CRANKING |
| Mixture | IDLE-CUT-OFF |
| Throttle | OPEN |
| Boost Pump | OFF |
| Fuel Selector | OFF |

| ENGINE POWER LOSS DURING TAKEOFF RUN |  |
| --- | --- |
| Throttle | IDLE |
| Brakes | APPLY |
| Wing Flaps | RETRACT |
| Mixture | IDLE-CUT-OFF |
| Ignition Switch | OFF |
| Master Switch | OFF |

| ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF |  |
| --- | --- |
| Airspeed | 72 KNOTS MINIMUM |
| Fuel Selector | SWITCH TANKS |
| Boost Pump | ON |
| Mixture | FULL RICH |
| Alternate Air | OPEN |
| If restart is unsuccessful, proceed with power off landing procedures |  |

| ENGINE POWER LOSS DURING FLIGHT |  |
| --- | --- |
| Fuel Selector | SWITCH TO TANK WITH FUEL |
| Boost Pump | ON |
| Mixture | RICH |
| Alternate Air | OPEN |
| When power is restored: |  |
| Alternate Air | CLOSED |
| Boost Pump | OFF |
| If power is NOT restored, prepare for POWER OFF LANDING |  |
| POWER OFF LANDING APPROACH |  |
| Trim for 79 KTS |  |
| Locate Suitable Landing Site |  |
| Establish Sprial Pattern |  |
| Position aircraft 1000Ft above terrain on downwind |  |
| When landing assured, slow to 72 KTS for shortest landing |  |
| EMERGENCY LANDING (GEAR UP OR DOWN) |  |
| When committed to landing |  |
| Landing Gear Selector | AS DESIRED Rough terrain, UP Suitable hard surface, DOWN |
| Throttle | CLOSE |
| Mixture | IDLE-CUT-OFF |
| Ignition | OFF |
| Master Switch | OFF |
| Fuel Selector | OFF |
| Seat Belts and Shoulder Harnesses | TIGHT |
| Touchdown | 72 KTS (Gear down) Slowest possible speed (Gear up) |

| TWO | PIPER ARROW IV EMERGENCY PROCEDURES | TWO |
| --- | --- | --- |

| ENGINE FIRE IN FLIGHT |  |
| --- | --- |
| Mixture | IDLE-CUT-OFF |
| Fuel Selector Valve | OFF |
| Master Swtich | OFF |
| Cabin Heat and Air | OFF (Except overhead vents) |
| Airspeed | 100 KTS OR GREATER |
| Find airspeed that will result in an incombustible mixture. |  |
| Forced Landing | EXECUTE |

| ELECTRICAL FIRE IN FLIGHT |  |
| --- | --- |
| Master Switch | OFF |
| Avionics Master Switch | OFF |
| All other switches (EXCEPT IGNITION) | OFF |
| Attitude Indicator | LEVEL (+/- 5 DEGREES) |
| Vents/Cabin Air/Heat | CLOSED |
| Fire Extinguisher | ACTIVATE (if available) |
| Land as soon as practical. If electrical equipment and continued flight is required... |  |
| Master Switch | ON |
| Circuit Breakers | CHECK BUT DO NOT RESET |
| Radios and Avionics | OFF |
| Avionics Master Switch | ON |
| Radio/Electrical Switches | ON ONE AT A TIME |
| Vents/Cabin Air/Heat | OPEN ONLY WHEN FIRE IS OUT |

| WING FIRE |  |
| --- | --- |
| Navigation Light Switch | OFF |
| Pitot Heat Switch | OFF |
| Sideslip | PERFORM TO KEEP FLAMES AWAY FROM FUEL TANKS |
| Flaps | USE ONLY AS REQUIRED FOR FINAL APPROACH AND LANDING |

| AMMETER SHOWS EXCESSIVE LOAD |  |
| --- | --- |
| Electrical Load | REDUCE |
| ALT Switch | OFF If loads are not reduced, continue to next step of this procedure |
| ALT Switch | ON |
| BAT Switch | AS REQUIRED Land as soon as practical and anticipate complete electrical system failure |

| ALT ANNUNICATOR ILLUMINATED |  |
| --- | --- |
| ALT Switch | OFF |
| ALT Circuit Breaker | RESET |
| Nonessential Electrical Equipment | OFF |
| ALT Switch | ON If power is NOT restored, continue to next step of this procedure |
| ALT Switch | OFF Land as soon as practical and anticpate complete electrical system failure |

| THREE | PIPER ARROW IV EMERGENCY PROCEDURES | THREE |
| --- | --- | --- |

| PROPELLER OVERSPEED |  |
| --- | --- |
| Throttle | RETARD |
| Oil Pressure | CHECK |
| Propeller Control | FULL DECREASE Set normal RPM if possible |
| Airspeed | REDUCE |
| Throttle | AS REQUIRED Maintain RPM below 2650 RPM redline |

| INADVERTANT ICING ENCOUNTER |  |
| --- | --- |
| Pitot Heat Switch | ON |
| Altitude | CHANGE Attempt to achieve OAT less conducive to icing conditions |
| Propeller Control | VARY or INCREASE Attempt to shed ice from propeller blades |
| Throttle | AS REQUIRED Maintain RPM below 2650 RPM redline |
| Landing | PLAN Use longest runway with best braking action conditions possible |
| Wing Flaps | UP Leave retracted for landing |
| Forward Slip | EXECUTE ON FINAL APPROACH If required for improved visibility |
| Approach Airspeed | 75 K MINIMUM |
| Landing Technique | LEVEL ATTITUDE With little or no flare |