| ONE | PIPER ARROW IV NORMAL PROCEDURES | ONE |
| --- | --- | --- |

| BEFORE STARTING ENGINE |  |
| --- | --- |
| Passenger Briefing | COMPLETE |
| Preflight Inspection | COMPLETE |
| Seats, Seat Belts, Shoulder Harnesses | ADJUSTED and LOCKED |
| Fuel Quantity and Quality | CHECKED |
| Fuel Shutoff Valve | LOWER TANK |
| Avionics Master Switch | OFF |
| All Electrical Switches | OFF |
| Circuit Breakers | CHECK |
| Alternate Air | OFF |
| Mixture | IDLE-CUT-OFF |
| Propeller | HIGH RPM |
| Throttle | CLOSED |
| Turn Coordinator | RED FLAG VISIBLE |
| Airspeed Indicator | ZERO |
| Altimeter | SET ELEVATION |
| VSI | LEVEL |

| STARTING ENGINE WHEN COLD |  |
| --- | --- |
| Throttle | ½ “ OPEN |
| Master Switch | ON |
| Boost Pump | ON |
| Mixture | FULL RICH to prime, then IDLE-CUT-OFF |
| Brakes | APPLIED |
| Propeller Area | CLEAR |
| Starter | ENGAGE |
| Mixture | FULL RICH when engine fires |
| Throttle | ADJUST |
| Oil Pressure | CHECK |

| STARTING ENGINE WHEN HOT |  |
| --- | --- |
| Throttle | ¼” OPEN |
| Master Switch | ON |
| Boost Pump | ON |
| Mixture | IDLE-CUT-OFF |
| Brakes | APPLIED |
| Propeller Area | CLEAR |
| Starter | ENGAGE |
| Mixture | FULL RICH SLOWLY when engine fires |
| Throttle | ADJUST |
| Oil Pressure | CHECK |

| STARTING ENGINE WHEN FLOODED |  |
| --- | --- |
| Throttle | OPEN FULL |
| Master Switch | ON |
| Boost Pump | OFF |
| Mixture | IDLE-CUT-OFF |
| Brakes | APPLIED |
| Propeller Area | CLEAR |
| Starter | ENGAGE |
| Mixture | FULL RICH SLOWLY when engine fires |
| Throttle | RETARD and ADJUST |
| Oil Pressure | CHECK |

| TWO | PIPER ARROW IV NORMAL PROCEDURES | TWO |
| --- | --- | --- |

| BEFORE TAXIING |  |
| --- | --- |
| Engine Warm-Up | 1000-1200 RPM |
| Boost Pump | OFF |
| Ammeter | CHECK |
| Internal/External Lights | ON AS REQUIRED |
| Avionics Master Switch | ON |
| Intercom | ON |
| Radios and Avionics | SET |
| Transponder | STANDBY with CODE SET |
| Altimeter | SET LOCAL BARO/NOTE ERROR |
| Attitude Indicator | CHECK |
| Heading Indicator | SET |
| Radio Check | OBTAIN |
| Autopilot | TEST |

| TAXIING |  |
| --- | --- |
| Brakes | CHECK |
| Steering | CHECK |
| Turn Coordinator Heading Indicator Magnetic Compass | CHECK DURING TURNS |
| Attitude Indicator | LEVEL (+/- 5 DEGREES) |

| BEFORE TAKEOFF / GROUND CHECK |  |
| --- | --- |
| Parking Brake/Toe Brakes | SET |
| Cabin Doors and Windows | CLOSED and LATCHED |
| Seats, Seat Belts, Shoulder Harnesses | SECURE |
| Flight Controls | FREE and CORRECT |
| Elevator Trim Tab | SET for TAKEOFF |
| Flight Instruments | CHECK and SET |
| Fuel Quantity | CHECK |
| Fuel Shutoff Valve | FULLER TANK |
| Mixture | FULL RICH (below 3000 feet, otherwise, lean for peak EGT) |
| Propeller | FULL INCREASE |
| Throttle | 2000 RPM |
| Magnetos | CHECK (175 RPM drop MAX, 50 RPM Differential) |
| Propeller | CYCLE (3 times first start of day, 1 time otherwise, MAX 500 RPM drop) |
| Alternate Air | CHECK |
| Vacuum Guage | CHECK (4.8” to 5.1” inclusive) |
| Engine Instruments and Ammeter | CHECK |
| Annunciator Panel | TEST |
| Throttle | RETARD, CHECK IDLE, then AS DESIRED |
| Friction Lock | ADJUST |
| Radios and Avionics | SET |
| Wing Flaps | SET for TAKEOFF (10 normal, 25 short/soft) |
| Parking Brake | RELEASE |

| THREE | PIPER ARROW IV NORMAL PROCEDURES | THREE |
| --- | --- | --- |

| IN POSITION |  |
| --- | --- |
| Boost Pump | ON |
| Landing Light | ON |
| Transponder | ON |
| Strobe Lights | ON |
| Autopilot | READY |
| Heading/Compass/Heading Bug | CHECK and SET |

| NORMAL TAKEOFF |  |
| --- | --- |
| Flaps | SET |
| Elevator Trim Tab | SET |
| Rotation Airspeed | 67-75 KTS |
| Control Wheel | ROTATE to climb attitude |

| SHORT FIELD TAKEOFF WITH OBSTACLE CLEARANCE |  |  |
| --- | --- | --- |
| Backup Gear Extender | OVERRIDE |  |
| Flaps | 25 degrees |  |
| Elevator Trim Tab | SET slightly NOSE UP |  |
| Accelerate to 50-60 KTS, depending on weight |  |  |
| Control Wheel | ROTATE |  |
| Accelerate to 55-65 KTS, depending on weight |  |  |
| Gear | UP after positive Rate of Climb established and no landing runway remains |  |
| Accelerate to Vx (77 KTS) and maintain until clear of obstacle |  |  |
| Flaps |  | 10 degrees after gear unsafe light extinguishes |
| Accelerate to Vy (87 KTS) |  |  |
| Flaps |  | UP |
| Backup Gear Extender |  | NORMAL |

| SOFT FIELD TAKEOFF |  |
| --- | --- |
| Backup Gear Extender | OVERRIDE |
| Flaps | 25 degrees |
| Elevator Trim Tab | SET slightly NOSE UP |
| Accelerate to 50-60 KTS, depending on weight |  |
| Control Wheel | ROTATE |
| Accelerate to 55-65 KTS in ground effect, depending on weight, then start climb |  |
| Gear | UP after positive Rate of Climb established and no landing runway remains |
| Accelerate to Vx (77 KTS) |  |
| Flaps | 10 degrees after gear unsafe light extinguishes |
| Accelerate to Vy (87 KTS) |  |
| Flaps | UP |
| Backup Gear Extender | NORMAL |

| FOUR | PIPER ARROW IV NORMAL PROCEDURES | FOUR |
| --- | --- | --- |

| AFTER TAKEOFF / BALKED LANDING / MISSED APPROACH |  |
| --- | --- |
| Flaps | UP |
| Gear | UP |
| Boost Pump | OFF (check fuel pressure) |
| Power | ADJUST (25”, 2500 RPM typical) |
| Engine and Flight Instruments | CHECK |
| Airspeed | Vx (77 KIAS) or Vy (87 KIAS) |
| Landing Light | OFF |

| CRUISE (REPEAT EVERY 15-30 MINUTES DURING CRUISE FLIGHT) |  |
| --- | --- |
| Power | 15-25”, 2100-2700 RPM |
| Elevator Trim | ADJUST |
| Mixture | LEAN as required |
| Boost Pump | ON |
| Fuel Shutoff Valve | SWITCH TANKS |
| Fuel Pressure | CHECK |
| Directional Gyro | CHECK/SET |
| Communication/Navigation Priorities | SET |

| IN RANGE (30 NM FROM DESTINATION) |  |
| --- | --- |
| ATIS/Weather | RECEIVED |
| Communication Priorities | SET |
| Navigation Priorities | SET |
| Missed Approach Navigation | SET |
| Directional Gyro and Heading Bug | CHECK/SET |
| Approach and Missed Approach Procedures | REVIEW |
| Alternate Approach Plates | ACCESSIBLE |

| DESCENT |  |
| --- | --- |
| Fuel Shutoff Valve | FULLER TANK |
| Power | AS DESIRED |
| Mixture | ADJUST for smooth operation |
| Autopilot | AS DESIRED |

| BEFORE LANDING |  |
| --- | --- |
| Seats, Seat Belts, Shoulder Harnesses | ADJUSTED and LOCKED |
| Internal/External Lights | AS DESIRED |
| Boost Pump | ON |
| Gear | DOWN and 3 GREEN (130 KTS MAX) |
| Flaps | SET (108 KTS MAX) |
| Mixture | FULL RICH |
| Propeller | HIGH RPM |

| AFTER LANDING |  |
| --- | --- |
| Flaps | UP |
| Boost Pump | OFF |
| Lights | AS DESIRED |
| Pitot Heat | OFF |
| Transponder | STANDBY |

| FIVE | PIPER ARROW IV NORMAL PROCEDURES | FIVE |
| --- | --- | --- |

| SECURING AIRPLANE |  |
| --- | --- |
| Parking Brake | SET |
| Lights | OFF |
| Avionics Master Switch | OFF |
| Mizture | IDLE-CUT-OFF |
| Magnetos | OFF |
| Master Switch | OFF |
| Control Wheel | SECURE WITH SEAT BELT |
| Times | RECORD |
| Pitot Tube Cover | INSTALL |
| Cabin | LOCK |
| Flight Plan (if VFR) | CLOSE |

| AIRCRAFT INFORMATION |  |
| --- | --- |
| Vs0 | 53 |
| Vs | 58 |
| Vx | 77 |
| Vx (gear down) | 70 |
| Vy | 87 |
| Vy (gear down) | 76 |
| Vfe | 108 |
| Va | 96 – 121 |
| Vno | 149 |
| Vlo (retraction) | 109 |
| Vlo (extension) | 130 |
| Vle (max) | 130 |
| Vne | 190 |
| Vref | 74 |
| Max Demo X-Wind | 17 |