| ONE | PIPER ARROW IV PREFLIGHT INSPECTION | ONE |
| --- | --- | --- |

| INTERIOR |  |
| --- | --- |
| Times | CHECK |
| Control Wheel | FREE |
| Avionics Master Switch | OFF |
| All Electrical Switches | OFF |
| Parking Brake | SET |
| Mixture | IDLE-CUT-OFF |
| Magnetos/Starter Switch | OFF |
| Master Switch | ON |
| Fuel Guages | CHECK QUANTITY |
| Annunciator Panel | TEST |
| Gear Indicators | 3 GREEN |
| Instrument Operation | CHECK |
| Vacuum Source Fail Light | ON |
| Master Switch | OFF |
| Fuel Selector | LOWER TANK |
| Standby Vacuum Source | OFF |
| Pitot and Static Systems | DRAIN |
| Primary Flight Controls | PROPER OPERATION |
| Flaps | CHECK 10/25/40, LEAVE DOWN |
| Trim | NEUTRAL |
| Required Papers | CHECK |

| RIGHT WING |  |
| --- | --- |
| Baggage Door | CLOSED |
| ALL ice, snow, frost | REMOVED |
| Flap and Aileron Hinges, Control Linkages, Wing Skin | OK |
| Wing Tip and Lights | CHECK |
| Leading Edge | CHECK |
| Tie Down | REMOVE |
| Fuel Vent | UNOBSTRUCTED |
| Right Fuel Tank Sample | CLEAN and CORRECT |
| Right Fuel Tank Quantity | CHECK |
| Chock | REMOVE |
| Main gear strut inflation | 2.5” (+/- ¼ inch) |
| Tire, Brake Assembly | OK |
| Fresh Air Inlet | UNOBSTRUCTED |

| TWO | PIPER ARROW IV PREFLIGHT INSPECTION | TWO |
| --- | --- | --- |

| NOSE |  |
| --- | --- |
| Windshield | CLEAN and INTACT |
| Right Side Cowling Fasteners | SECURE |
| Engine Oil Level | CHEC (8 Qts MAX, 5 Qts MIN) |
| Air Inlets | CLEAR |
| Alternator Belt | CHECK TENSION |
| Propeller, Spinner | CHECK for leaks, nicks/cracks |
| Exhaust Stacks | SECURE |
| Nose Strut Inflation | 2.75” (+/- ¼ inch) |
| Left Side Cowling Fasteners | SECURE |
| Gascolator Drain Sample | CLEAN and CORRECT |
| Belly Skin Condition | SIGHT TO TAIL OK |
| Chock | REMOVE |
| LEFT WING |  |
| ALL Ice, Snow, Frost | REMOVE |
| Fresh Air Vent | UNOBSTRUCTED |
| Main Gear Strut Inflation | 2.5” (+/- ¼ inch) |
| Chock | REMOVE |
| Left Fuel Tank Sample | CLEAN and CORRECT |
| Left Fuel Tank Quantity | CHECK |
| Fuel Vent | UNOBSTRUCTED |
| Tie Down | REMOVE |
| Leading Edge | CHECK |
| Stall Warning Vane | FREE MOVEMENT |
| Wing Tip and Lights | CHEC |
| Pitot/Static Head | REMOVE COVER, PORTS CLEAR |
| Aileron and Flap Hinges, Control Linkages, Wing Skin | OK |
| FUSELAGE |  |
| Antennas | CHECK |
| Empennage | CLEAR of Ice, Snow, Frost |
| Fresh Air Inlet | UNOBSTRUCTED |
| Stabilator and Trim Tab | CHECK |
| Tail Tie Down | REMOVE |
| Inspection Plates | SECURE |
| EXTERIOR SYSTEMS |  |
| Master Switch | ON |
| Cockpit Lighting | CHECK |
| Exterior Lights | CHECK |
| Stall Warning | CHECK |
| Pitot Heat | CHECK |
| All Electrical Switches | OFF |
| PREFLIGHT INSPECTION COMPLETE |  |